POST-PRODUCTION TERMS

480i

A video mode with a resolution of either 720x480 or 704x480 pixels. The 480 part refers to the number of horizontal lines and the i part refers to interlacing. The interlaced signal contains two fields of a video frame captured at two different times. This enhances motion perception to the viewer, 480i mode is used in most standard definition televisions.

480p
A video mode with a resolution of either 640x480 or 854x480 pixels. The 480 part refers to the number of horizontal lines and the p refers to progressive scanning, the opposite of interlacing. It is often offered on high definition televisions but is not considered a high definition mode itself.

4K
A term used to describe video that has approximately 4000 horizontal pixels. The actual number will depend on the aspect ratio but common resolutions are 4096x3112, 4096x1714, 3996x2160, 3656x2664. The term “4K” can also be used to describe Ultra High Definition (UHD) which has a resolution of 3840x2160.

720p
High definition video with a progressive scan resolution of 1280x720 pixels.

1080i

A high definition interlaced video mode consisting of 1080 horizontal lines. The i refers to interlacing. The interlaced signal contains two fields of a video frame captured at two different times. This enhances motion perception to the viewer. The term 1080P refers to video that was shot at a resolution of 1920x1080.

1080p
A high definition progressive video mode consisting of 1080 horizontal lines. The term 1080P refers to video that was shot at a resolution of 1920x1080.

A

Alpha channel

An image channel in addition to the R, G, and B color channels that is used to store transparency information for compositing. Black represents 100 percent transparent, and white represents 100 percent opaque.

Ambience

A type of sound that includes background room noise, traffic noise, and atmospheric sound effects.

Anamorphic widescreen

A process by which a widescreen image is compressed horizontally to fit into a storage medium with a narrower aspect ratio. Compatible playback equipment can then re-expand the horizontal dimension to show the original widescreen image.

Animation

The process of changing any number of variables such as color, audio levels, or other effects over time using keyframes.

Animated graphic

A picture that has more than one frame and depicts movement.

Aspect ratio

The ratio of the width of an image to its height on any viewing screen. Standard TV has an aspect ratio of 4:3; HDTV’s is 16:9.

Audio mixing

The process of adjusting the volume levels of all audio clips in an edited sequence, including the production audio, music, sound effects, voice-overs, and additional background ambience, to turn all of these sounds into a harmonious whole.

Audio sample rate

The rate or frequency at which a sound is sampled to digitize it. 48 kHz is the standard sampling rate for digital audio; CD audio is sampled at 44.1 kHz.

Audio waveform

A graphical representation of the amplitude (loudness) of a sound over a period of time.

B

Bed or music bed

Background music used underneath narration.

B-roll

A term used to describe alternate footage that intercuts with the primary soundtrack used in a program to help tell the story, or to cover flaws. B-roll is also referred to as cut away shots.

Buy out

Music or music libraries in which a one-time fee enables the buyer to legally use the music in many productions without paying additional licensing or "needle drop" fees.

C

Capture or import

The process of digitizing media into an editing computer for post-production.

Channel 
One of several grayscale components used to make up a color image. RGB images are made up of red, green and blue channels, with an optional alpha channel for transparency.

Chroma key

A compositing process that allows a selected color in an image to be made transparent. Commonly used for green screen compositing.

Clip

Media files that may consist of video, audio, graphics, or any similar content that can be imported into a non-linear editing system.

closed captioning

Closed captions, or subtitles, allow a textual representation of a video to be displayed during playback.  Useful for the hearing impaired, foreign language videos and in noisy environments like tradeshows.

Codec

Short for compression/decompression. A device or software that enables compression or decompression of digital video.

Color space 
A mathematical model of color. Color spaces differ in how they model color - for example, RGB creates color from mixtures of red green and blue and CMYK creates color from mixtures of cyan, magenta, yellow and black.  Video production uses the RGB color space.

Color grading/color correction

A process of altering and enhancing the image color in a video clip. Generally, color correction referrers to matching the color of clips used in an edited program. Color grading also includes any generation of artistic color effects.

Color depth

The possible range of colors that can be used in a movie or image. In computer graphics, there are usually four choices: grayscale, 8-bit, 16-bit, and 24-bit. Higher color depths provide a wider range of colors but also require more disk space for a given image size. Broadcast video is generally 24-bit, with 8 bits of color information per channel.

Color matte

A clip containing solid color created as a generated item in the effects. Mattes are used in video to combine two or more image elements into a single, final image. Usually, mattes are used to combine a foreground image (such as talents on a set) with a different/separate background image.

Compositing

The process of combining two or more video or electronic images into a single frame. This term can also describe the process of creating various video effects.

Compression

The process by which video, graphics, and audio files are reduced in size. The reduction in the size of a video file through the removal of redundant image data is referred to as a lossy compression scheme. A lossless compression scheme uses a mathematical process and reduces the file size by consolidating the redundant information without discarding it. See also codec.

Container

Containers within a video file "contain" the various components of a video: the stream of images, the sound, and anything else.

Contrast

The difference between the lightest and darkest values in an image. High-contrast images have a large range of values from the darkest shadow to the lightest highlight. Low-contrast images have a more narrow range of values, resulting in a “flatter” look.

Cross fade

A transition between two audio clips where one sound is faded out while the other is faded in. Used to make the transition between two audio cuts less noticeable.

Cut

The simplest type of edit where one clip ends and the next begins without any transition.

Cutaway

A shot that is related to the current subject and occurs in the same time frame; for instance, an interviewer’s reaction to what is being said in an interview or a shot to cover a technically bad moment.

D

Data rate

The speed at which data can be transferred, often described in megabytes per second (MB/sec).  The higher a video file's data rate, the higher quality it will be, but it will require more system resources (processor speed, hard disk space, and performance).  Some codecs allow you to specify a maximum data rate for a movie during capture.

Decibel (dB)

A unit of measure for the loudness of audio.

De-Interlace

is the process of converting interlaced video (video comprised of two separate, alternating fields), into a non-interlaced, or progressive, form.

Decode
The process of reading data in one format and outputting it in another, usually for the purpose of converting from a compressed format to one capable of being displayed on a monitor. See also encode.

Deliverable

The final video file, encoded with the proper specs for your desired playback method.

Desaturate

To remove color from a clip. 100 percent desaturation results in a grayscale image.

Digital

Denotes data stored or transmitted as a sequence of ones and zeros.

Digital video

Video that has been captured, manipulated, and stored using a digital format, which can be easily imported into a non-linear editing system.

Digitize

To convert an analog video signal into a digital video format. A method of capturing video. See also capture.

Dissolve

A transition between two video clips where the first one fades down at the same time the second one fades up.

Drop-frame timecode
Timecode that is modified to remain in sync when 29.97 fps NTSC video is broadcast at 30 fps. In order to retain accuracy, the first two timecode frames of every minute are dropped, with the exception of every tenth minute.

Drop shadow

An effect that creates an artificial shadow behind an image or text.

Duration

The length of a clip or a sequence from its In to its Out point, or the length of time that it takes that piece of video to play.

E

Editing

The process of manipulating and rearranging video shots to create a new work. Editing is usually considered to be one part of the post-production process — other post-production tasks include titling, color correction, sound mixing, etc.

Edit point

(1) Defines what part of a clip you want to use in an edited sequence. Edit points include In points, which specify the beginning of a section of a clip or sequence, and Out points, which specify the end of a section of a clip or sequence.

(2) The point in the Timeline of an edited sequence where the Out point of one clip meets the In point of the next clip.

EDL (Edit Decision List)

A text file that uses the source time code of clips to sequentially list all of the edits that make up a sequence. EDLs are used to move a project from one editing application to another, or to coordinate the assembly of a program in a tape-based online editing facility.

Effects

A general term used to describe all post-production actions that go beyond cuts-only editing. Effects can be applied to audio, video, text, graphics and animations.

Export

Outputting a final video, graphic, animation or audio file. This is accomplished by using codecs, which lets you translate the current file format into a number of different formats.

F

Fade in / fade out

An effect applied to a video clip during post-production in which the picture gradually transitions to or from black.

Feather
A technique used for blurring the edges of a mask on a video clip or graphic during post-production.

Field
Interlaced video is split into two fields: one comprising the odd-numbered scan lines, and the other comprising the even lines. There are 60 fields for every second of NTSC video.

Field dominance

The choice of whether the even or odd field will be displayed on the monitor first.

File extension

A filename extension is a suffix (separated from the base filename by a dot or space) to the name of a computer file applied to indicate the encoding (file format) of its contents or usage.

Filters

Effects that you can apply to video and audio clips or group of clips during post-production that change some aspect of the clip content.

Finishing

The process of fine-tuning the sequence audio and video levels and preparing the sequence for output for its final deliverable.

Frames per second (fps)
The number of frames played every second. The standard video frame rate is 29.97 fps, although other frame rates are used.  Video shot in Europe (PAL) is usually 25 fps.

Frequency

The number of times a sound or signal vibrates each second, measured in cycles per second, or hertz.

G

Gain

In video, the level of white in a video picture; in audio, the loudness of an audio signal.

H

Headroom 
The space between the top of a character's head and the top of the frame.

High definition

Video that is created at a higher resolution that standard definition (720x486). Typical high definition resolutions include 1280x720 and 1920x1080.

Hue

A specific color or pigment, such as red.

I

In point

The beginning point of a video clip that you want to use as part of your video.

Interlaced video

A video scanning method that first scans the odd picture lines (field 1) and then scans the even picture lines (field 2), which merges them into one single frame of video.

Intercut

A technique where the editor cuts back and forth between two separate scenes as they play out. This is commonly used to give the impression that the actions are occurring simultaneously.

J

JPEG (Joint Photographic Experts Group) A popular image file format that lets you create highly compressed graphics files. The amount of compression can vary. Less compression results in a higher-quality image.

Jump cut

An editing cut in which the action does not completely match that of the preceding shot, causing characters to "jump" to a slightly different position. Often jump cuts are covered with b-roll footage.

K

Key frame
A frame that contains a record of specific settings (e.g. scale, rotation, brightness). By setting multiple keyframes, you can adjust these parameters as the video plays to animate certain aspects.

Keying

The process of dropping out a specific area of an image, such as its background, so that the image can be composited with another. You can key out information in a clip based on brightness and darkness, or color.

L

Letterbox

A format in which widescreen video is displayed to fit within a standard 4 x 3 monitor, putting black at the top and bottom of the picture.

Log

A document, completed during the shoot of a show, which includes detailed information, such the raw file numbers of your source material. Also included are any comments on each shot captured.

Lower third

Lines of text used to identify a person, place, or thing in a clip. Lower thirds are traditionally used to display an on-screen talent’s name, along with other pertinent information such as business title, city/state, etc.

M

Mask

An image or clip used to define areas of transparency in another clip. Similar to an alpha channel. A mask enables part of an image to be used while masking out the rest of the image.

Metadata

Metadata is data, within a file, that describes other data. Basic examples of metadata include author, date created and date modified and file size.

Media file or media asset

A generic term for captured or acquired elements such as movies, animation, sounds, graphics, and photos.

Midtones

The middle brightness range of an image. Not the very brightest part, nor the very darkest part.

Mono audio

A single track of audio.

Montage
A self-contained sequence of short shots or images used to portray facts, mood or actions, that is often used to indicate the passing of time.

Motion blur

An effect that blurs any clip with keyframed motion applied to it, similar to blurred motion recorded by a camera.

MPEG (Moving Picture Experts Group)

A group of compression standards for video and audio, which includes MPEG-1, MPEG-2, MPEG-3 (referred to as MP3, an audio only format), and MPEG-4.

MXF

Material eXchange Format A professional, cross-platform container format for video, audio and metadata. MXF is the file format used by AVID editing systems.

N

Natural sound

The ambient sound used from a source videotape.

Non-drop-frame timecode

Timecode that is modified to remain in sync at 30 frames per second. In almost all cases, timecode is non-drop frame.

Non-linear editing

An editing method in which edits can be performed at any time, in any order. Access is random, which means that the system can jump to specific pieces of data without having to look through the whole footage to find it.

Non-interlaced video

The standard representation of images on a computer, also referred to as “progressive scan.” The monitor displays the image by drawing each line, continuously one after the other, from top to bottom.

Non-square pixel

A pixel whose height is different from its width. An NTSC pixel is taller than it is wide, and a PAL pixel is wider than it is tall.

NTSC (National Television Systems Committee) Standard of color TV broadcasting used mainly in North America, Mexico, and Japan.

O

Opacity

The degree to which an image is transparent, allowing images behind to show through.

Out point

The ending edit point of a video clip.

P

PAL (Phase Alternating Line)

The European color TV broadcasting standard.

Phase

An attribute of color perception, also known as hue.

PICT

The native still-image file format for Macintosh developed by Apple. PICT files can contain both vector images and bitmap images, as well as text and an alpha channel.

Pillarbox
The black bars displayed at the sides when a 4:3 image is shown in widescreen.

Picture in picture (PIP)
An effect where a small window of footage is superimposed over a larger window and the two play at the same time.

Pixel

Short for “picture element,” one dot in a video or still image.

Pixel aspect ratio

The width-to-height ratio for the pixels that compose an image. Pixels on computer screens and in high definition video signals are square (1:1 ratio).

Pixelated or pixelation
The display of large, blocky pixels in an image caused by over-enlarging it.

Pre-roll ad

A pre-roll ad is a promotional video message that plays before the content the user has selected.

Production music or stock music

Production music is the name given to recorded music produced and owned by production music libraries and licensed to customers for use in film, television, radio and other media.

Post-production

The process of arranging acquired video, audio, animation and graphic assets into a cohesive, compelling story.

Processed audio

Audio that has been manipulated to create the best sound playback. Breaths, lip smacks and unwanted ambient sounds are removed and the overall decibel level is increased.

Q

QuickTime Apple’s cross-platform multimedia technology. Quicktime (.mov) is used for editing, compositing, CD-ROM, web video, and more.

R

Raw audio

Audio and/or narration as it was recorded.

Reference tone
An audio tone of fixed frequency and amplitude that occurs at the beginning of a tape or reel, allowing the operator to set the correct audio level for playback.

Render

To process video and audio with any applied effects, such as transitions or filters. Many visual effects need to be rendered in order to be played back properly.

RGB

An abbreviation for red, green, and blue, which are the three primary colors that make up a color image.

Rough cut

The first cut created by an editor, intended to show how selected cuts will flow together to make the story.

S

Saturation

The purity of color. As saturation is decreased, the color moves toward pastel, then toward white.

SECAM (Séquentiel Couleur à Mémoire)

The French television standard for playback. Similar to PAL, the playback rate is 25 fps.

Selects

Specific quotes or portions of quotes, chosen by a producer, to be used in a video project. Selects are most commonly chosen using a transcript.

Sequence

An edited assembly of video, audio, or graphics clips. A sequence may contain as many clips as needed to tell the story. A sequence can contain an entire edited program or be limited to a single scene.

SMPTE (Society of Motion Picture and Television Engineers)

The organization responsible for establishing various broadcast video standards like the SMPTE standard time code for video playback.

Sound bite

A short excerpt taken from an interview clip.

Square pixel

A pixel that has the same height as width. Computer monitors have square pixels, but NTSC and PAL video do not.

Standard definition

A television system that uses a resolution that is not considered to be high-definition television (HDTV 720p, 1080i, and 1080p). Most standard definition signals are created at a resolution of 720x480 or 720x486.

Stereo audio

Sound that is separated into two channels, one carrying the sounds for the right ear and one for the left ear. Stereo pairs are linked and are always edited together. Audio level changes are automatically made to both channels at the same time. A pair of audio items may have their stereo pairing enabled or disabled at any time.

Stock footage

Common footage, shot by professionals that can be purchased and used for video projects.

Streaming media/video

Multimedia that is constantly received by, and normally displayed to, the end-user while it is being delivered by the provider. Internet video and/or audio clips that can play directly over the Internet, without needing to be downloaded first onto a computer.

Sweetening

The process of creating a high-quality, final sound mix by polishing sound and narrations levels, music, and sound effects.

Sync or audio video sync

The relationship between the image of a sound being made in a video clip (for example, a person talking) and the corresponding sound in an audio clip.

T

Tail

The end frames of a clip.

Transcript

A document created by turning oral language into written form. Watching/listening to an  audio or video recording and then typing it as a written transcript.

TIFF (Tagged Image File Format) A widely used bitmapped graphics file format that handles monochrome, grayscale, and 8- and 24-bit color. There are two types of TIFF images, one with an alpha channel and one without.

Timecode

A numbering system of electronic signals laid onto each frame of videotape that is used to identify specific frames of video. Each frame of video is labeled with hours, minutes, seconds, and frames (01:00:00:00).

Timeline
A visual representation of a movie over time, consisting of video clips laid horizontally across the screen. This is a common interface in non-linear video editing applications.

Titling

The use of on-screen text as credits, captions, or any other alphanumeric communication to video viewers. Used to depict a talent’s name and other vital information during their initial time on screen.

Transition

A visual or audio effect applied between two edit points, such as a video cross dissolve or an audio cross fade.

U

Underscan

Special video monitor that can reduce the size of the video image so the four outer frame edges can be viewed in their entirety.

V

Video format

The file type of a video. Different video formats are used by different programs and/or operating systems. The most commonly used formats for digital video are .mov (Quicktime) and .wmv (Windows).

Video on demand (VOD)

Permits a viewer to browse an online catalogue, to watch trailers and to then select a selected recording for playback

Video script

A two-column document containing all audio cues and narration on the right side, with all visual cues adjacent to the audio on the left side.

Video sharing networks

Internet websites such as YouTube and Vimeo that allow web browsers to share web video content with other users. Video sharing networks greatly contribute to the success of viral marketing and web video marketing in general.

Voiceover (VO)
Narration accompanying picture, heard above background sound or music, without narrator seen on camera. VO is typically applied to edited visual during post-production.

Voice talent

A trained, professional narrator or actor who is usually hired through a talent agency.

W

Web analytics

The use of metadata and other composite information collected from websites to determine the behavior of internet web browsers. Data is often collected through logfile analysis or page tagging and includes traffic reports, email response rates, direct marketing campaigns, and user performance data. All information is collected to provide better business solutions and increase the effectiveness of online marketing.

Widescreen 
A format in which the width-to-height ratio of the frame is greater than 4:3, so that it is significantly wider than it is tall.

Window Media File (.wmf)

An image file format originally designed for Microsoft Windows in the 1990s. WMV files are ideal for playback on PC-based computers.

Window burn or window dub

Visual timecode and keycode information superimposed onto video frames. It usually appears on a strip at the bottom or top of the frame, providing code information to the editor without obscuring any of the picture.

wipe

A type of transition that uses a moving edge to progressively erase the current clip to reveal the next clip.

X

x axis

Refers to the x coordinate in Cartesian geometry. The x coordinate describes horizontal placement in motion effects

.

Y

y axis

Refers to the y coordinate in Cartesian geometry. The y coordinate describes vertical placement in motion effects.

Z

z axis

Refers to the z coordinate in Cartesian geometry. The z coordinate describes perpendicular placement in motion effects.

1-18-18