Production Terms

The following terms are associated with the production phase of a video’s creation:

180 degree rule (action axis)

A production concept that, once a spatial relationship has been confirmed with the establishing shot, no close-up will cross the imaginary line drawn between those two actors until a new line (or axis) has been established, usually through another establishing shot.

A

Action

The producer or director’s cue to the talent(s) to begin performing.

Action safe area

A region of the screen where elements are guaranteed to be visible. The action safe area is larger than the title-safe area and runs right up to the edge of the viewable area.

Ambience

A type of sound that includes background room noise, traffic noise, and atmospheric sound effects.

Audio Technician

The Audio Tech is responsible for all audio aspects during a recording session, either on a production set or in an audio recording studio. The audio tech places mics on/near the talent, makes sure a strong, clean signal is getting to the recording device and monitors the audio during the recording

.

B

Back light (hair light)

A light mounted behind a subject to illuminate their hair and shoulders without affecting their front. Backlighting helps to separate a talent from their background. It is also called a rim light.

Bars and tone

A series of vertical bars of specific colors and an audio tone used to calibrate the audio and video signals coming from a camera to ensure consistent appearance and sound on different TV monitors.

Blocking
Plotting the placement and movement of the actors, camera and microphone in a scene prior to shooting the scene.

Boom or boom pole

A telescopic arm used for mounting a microphone. Also referred to as a boom pole or fish pole.

B-roll

A term used to describe alternate footage that intercuts with the primary footage used in a program to help tell the story, or to cover flaws. B-roll is also referred to as cutaway shots.

C

Clapboard

A whiteboard or chalk slate for documenting the show, scene and take, combined with a clap stick, which will help an editor establish/maintain audio and video synchronization.

Clip

A small section of a larger video shoot. A recorded segment starting from when the producer/director says “action” to the time they say “cut.”

Close-up

A tightly framed camera shot in which the talent is viewed at close range, appearing large and dominant on screen. Pulled back slightly is a "medium close-up" while zoomed in very close is an "extreme close-up (ECU or XCU).

Closed captioning

Closed captions or subtitles allow a textual representation of a video to be displayed during playback.  Useful for the hearing impaired, foreign language videos and in noisy environments like tradeshows.

Composition

Visual makeup of a video picture, including such variables as balance, framing, field of view, texture – all aesthetic considerations. Combined qualities form image that's pleasing to view, and effectively communicates.

Consent  form

A contract that gives you legal permission to use the image of the person who has signed the form for commercial and non-commercial purposes.

Continuity
The process of maintaining the consistency of the plot, characters, time period, objects, places and events of the film in order to maintain the audience's suspension of disbelief.

Cut

The producer or director’s cue to stop recording.

Cutaway

A shot that is related to the current subject and occurs in the same time frame; for example, an interviewer’s reaction to what is being said in an interview or a shot to cover a technically bad moment.

D

Digital video

Video that has been captured, manipulated, and stored using a digital format, which can be easily imported into a non-linear editing system.

Director

The production crew member who is primarily responsible for the storytelling, creative decisions and acting of the video. The director is responsible for overall success of a video production shoot.

Director’s monitor

The monitor being viewed by the director during a shoot. The Director’s monitor is usually a direct feed from the primary camera.

Director of Photography (DP)

The production crew member responsible for aspects of the imagry. The DP lights the set and ensures that high quality video is acquired.

Dolly right or dolly left

Camera movement to the left or right.

E/F

Fill light

A fill light floods light onto the talent the opposing side from the key light. It should be more diffused than the key light, allowing the shadows to stay on the subjects face but brightening them up enough to let the detail show through.

Filters

Transparent or translucent optical elements which are either attached to the front of the lens or included as part of the lens housing. Filters alter the properties of light before it reaches the cameras recording electronics. Filters can be used to correct problems with light or to create certain effects.

Framing

Composing a shot for the best presentation of the talent, taking into consideration the size of the talent in the frame and how it is centered.

G

Gel

Colored material placed in front of a light source to alter its hue. Useful for correcting mismatches in lighting, as in scenes lit by both daylight and artificial light.

Green screen 
A green background that the subject stands in front of that will later be replaced with another background in post-production.

Grip

The production crew member responsible for the set lighting and assisting the DP as needed.

H

Headroom 
The space between the top of a character's head and the top of the frame.

I/J

Jib arm
A counterweighted support arm mounted on a tripod or dolly to provide an increased range of camera motion. A jib arm is also called a crane.

K

Keeper take

The best shot or take for any given scene, usually determined by the producer or director.

Key light

Principal illumination source on a subject or scene, normally positioned slightly off center and angled to provide shadow detail.

Kicker

Also called a hair light or a rim light.  Placed behind the talent separate them from the background.

L

Lavaliere mic or lav mic
A small, usually wireless, microphone that can be hidden in clothing. The mic is attached to a transmitter which sends the audio signal to a receiver. That signal is fed into a recording device.

Load-in

A defined time, set by the producer, when the video crew can start carrying all of their gear into a shoot location.

Log (logarithmic) video

A video recording format used in high contrast and/or highly shadowed situations. Video shot in a Log format requires color correction but can provide the most flexibility to adjust a video file during post-production.

Logging

The process of entering detailed information, including the In and Out points from your source material, log notes, and so on, in preparation for a clip to be captured.

Lower third

Lines of text used to identify a person, place, or thing in a clip. Lower thirds are traditionally used to display an on-screen talent’s name, along with other pertinent information such as business title, city/state, etc.

M

Master shot

A single, long shot of some dramatic action from which shorter cuts such as close-ups and medium shots are taken in order to fill out the story.

Medium (mid) shot

A camera direction in which the subject is framed from the waist up.

N

Natural sound

The ambient sound used from a source videotape.

O/P

Pan

To move a camera head left or right without changing its position.

Paper edit

A technique of pre-editing.  A list of edits made entirely on paper by viewing Window Dub copies of the original camera files or pulled from transcripts of the original files.

Pickups
Reshooting certain shots to augment footage already recorded.

Point of view (POV)

Shot perspective whereby the camera assumes the talents’ view, and thus viewers see what the subject sees as if through their eyes.

Producer

The person who makes the video happen. The producer oversees all aspects of a video production from conceptualization to completion.

Production

The process of executing on the pre-production process. It is the act of creating the visual and audio elements needed to assemble your video.

R

Reaction (or react) shot
A shot which cuts away from the main scene in order to show the reaction of a character to it.

Rec 709
The basic color space used for high definition video. Rec 709 is the preferred color space for most basic shooting situations such as basic interviews and green screen recording.

Reference tone
An audio tone of fixed frequency and amplitude that occurs at the beginning of a tape or reel, allowing the operator to set the correct audio level for playback.

Rim light
A hard backlight on the talent used to separate them from the background and to add visual interest.

Room tone
Background sound recorded during production for the purpose of enabling the seamless modification and removal of audio in post-production. See also ambience.

Rule of thirds

Composition consideration suggesting that a picture appeals most with its primary point of interest appearing off-center. With screen divided into thirds vertically and horizontally, important elements should be targeted wherever imaginary lines cross.

Run and gun

A video production (shooting) style in which footage is shot with a minimal amount of gear, crew and time.

S

Scouting

Searching for an ideal setting in which to shoot a video or a specific scene within a video

Script

A two-column document containing a video’s narrative and additional audio clues on the right side and all visuals and visual clues on the left side.

Selects

Specific quotes or portions of quotes, chosen by a producer, to be used in a video project. Selects are most commonly selected using a transcript.

Shotgun mic
A highly directional microphone commonly used to record production sound. A shotgun mic is used with a boom (or fish) pole.

Slate
A visual and/or audio identifier recorded before the start of a shot intended to communicate important details to the editor. A slate may also include a clapboard for audio synchronization or a time code display.

Slider

A device comprised of bracketed tracks that mounts to tripod legs or a light stand. A slider has a moveable camera carriage, providing smooth camera movement.

Sound stage

A soundproof-hangar-like structure, building, or room, used for video production image acquisition, or shoting. Also called a studio.

Storyboard

A series of pictures that summarizes the content, action, and flow of a proposed project.

Strike
The process of breaking down a camera position, location or set.

Stylist

The person on-set who is responsible for the talents’ make-up hair and clothing.

T

Tally light

Automatic indicators on camera front and within viewfinder that signal recording in progress – seen by both the talent(s) and operator.

Teleprompter (prompter)

Electronic device that projects and advances text on mirror directly in front of camera's lens, allowing talent to read their lines while appearing to maintain eye contact with viewers.

Transcript

A document created by turning oral language into written form. Watching/listening to an audio or video recording and then typing it as a written transcript.

Three-point lighting

Basic lighting approach employing key, back, and fill lights to illuminate subject with sense of depth and texture.

Tilt

To pivot the camera head up and down, which causes the image to move up or down in the frame.

Title safe area

Part of the video image that is guaranteed to be visible on televisions. The title safe area is the inner 80 percent of the screen. To prevent text in your video from being hidden by the edge of a TV set, you should restrict any titles or text to the title safe area.

Truck in or truck out

A camera move in which the entire camera is moved closer to or further from the talent. The effect may appear the same as zooming, which reduces and magnifies the image, but dollying in or out maintains perspective while changing picture size.

Two-shot

Camera view including two subjects, generally applicable to interview situations.

U/V

Voice over (VO)
Narration accompanying picture, heard above background sound or music, without narrator seen on camera. VO is typically applied to edited visual during post-production.

Voice talent

A trained, professional narrator or actor who is usually hired through a talent agency.

W

White balance

A color camera function which determines how much red, green and blue is required to produce a normal-looking white. Shots made with improper white balance will have an abnormal color tint.

Wide angle

Camera lens with short focal length and broad horizontal field of view. Opposite of telephoto, supports viewer perspective and tends to reinforce perception of depth.

Wind screen

Sponge-like microphone shield, thwarts undesirable noise from wind and rapid mike movement.

Z

Zebras

A feature on professional video cameras that allows you to see where certain areas of brightness in the frame that may be "blown out," or overexposed. When this happens at a certain level the Zebra Stripes show up on that area to indicate that it may be too bright.

Zoom

Variance of focal length, bringing subject into and out of close-up range. Lens capability permits change from wide-angle to telephoto, or vice versa, in one continuous move. "Zoom in" and "zoom out" are common terms.

1-20-18